SPS Series Single Output Power Supply Technical Specifications

The instrument must be operated continuously for more than 30 minutes at the specified operating temperature to achieve the following specifications:

| Specifications | SPS3051 | SPS6051 | SPS3081 | SPS6081 | SPS8081 |
| --- | --- | --- | --- | --- | --- |
| Input Characteristics |  |  |  |  |  |
| Supply Voltage | 110Vac±10% or 220Vac±10% (please refer the power information on the back cover) |  | 85V-265V |  |  |
| Input Frequency | 50~60Hz |  | 45~65Hz |  |  |
| Full Input Current | ≤3A | ≤3A | ≤3A | ≤3A | ≤3A |
| No-Load Input Current | ≤300mA | ≤300mA | ≤300mA | ≤300mA | ≤300mA |
| Input Fuse | 250V,F5A | 250V,F5A | 250V,F5A | 250V,F5A | 250V,F5A |
| Rated Output |  |  |  |  |  |
| Voltage | 0~30V | 0~60V | 0~30V | 0~60V | 0~80V |
| Current | 0~5A | 0~5A | 0~8A | 0~8A | 0~8A |
| Power | 150W | 150W | 120W | 120W | 120W |
| Efficiency(110Vac,rated load) | 82% | 82% | 82% | 82% | 82% |
| Efficiency(220Vac,rated load) | 88% | 88% | 88% | 88% | 88% |
| Regulation(CV) |  |  |  |  |  |
| Load | ≤30mV | ≤30mV | ≤30mV | ≤30mV | ≤30mV |
| Power | ≤20mV | ≤20mV | ≤20mV | ≤20mV | ≤20mV |
| Regulation(CC) |  |  |  |  |  |
| Load | ≤30mV | ≤30mA | ≤30mA | ≤30mV | ≤30mV |
| Power | ≤20mV | ≤20mA | ≤20mA | ≤20mV | ≤20mV |
| Ripple & Noise (Noise bandwidth 20MHz, ripple bandwidth 1MHz, connect 10uF electrolytic capacitor in parallel with 0.1uF ceramic capacitor to the output terminal for testing) |  |  |  |  |  |
| Voltage(Vp-p) | ≤30mV | ≤30mV | ≤30mV | ≤30mV | ≤30mV |
| Voltage(rms) | ≤5mV | ≤5mV | ≤5mV | ≤10mV | ≤10mV |
| Current(Ap-p) | ≤10mA | ≤10mA | ≤10mA | ≤20mA | ≤20mA |
| Setting resolution |  |  |  |  |  |
| Voltage | 10mV | 10mV | 10mV | 10mV | 10mV |
| Current | 1mA | 1mA | 1mA | 1mA | 1mA |
| Readback Resolution |  |  |  |  |  |
| Voltage | 10mV | 10mV | 10mV | 10mV | 10mV |
| Current | 1mA | 1mA | 1mA | 1mA | 1mA |
| Setting resolution |  |  |  |  |  |
| Voltage | ≤0.05%+20mV | ≤0.05%+20mV | ≤0.1%+20mV | ≤0.1%+20mV | ≤0.1%+20mV |
| Current | ≤0.05%+20mA | ≤0.05%+20mA | ≤0.2%+20mA | ≤0.1%+20mA | ≤0.1%+20mA |
| Readback Resolution |  |  |  |  |  |
| Voltage | ≤0.05%+20mV | ≤0.05%+20mV | ≤0.1%+20mV | ≤0.1%+20mV | ≤0.1%+20mV |
| Current | ≤0.05%+20mA | ≤0.05%+20mA | ≤0.2%+20mA | ≤0.1%+20mA | ≤0.1%+20mA |
| Response time |  |  |  |  |  |
| Transient recovery time (50%~100% rated load) | ≤1.0ms |  |  |  |  |
| Protection function |  |  |  |  |  |
| OVP | 0~32V | 0~62V | 0~32V | 0~62V | 0~82V |
| OCP | 0~5.2A | 0~5.2A | 0~8.2A | 0~8.2A | 0~8.2A |
| OTP | 85℃ | 85℃ | 85℃ | 85℃ | 85℃ |
| Temperature coefficient of output |  |  |  |  |  |
| Voltage | 100ppm/℃ |  |  |  |  |
| Current | 200ppm/℃ |  |  |  |  |
| Temperature coefficient of readback value |  |  |  |  |  |
| Voltage | 100ppm/℃ |  |  |  |  |
| Current | 200ppm/℃ |  |  |  |  |
| Display |  |  |  |  |  |
| Display Type | 2.8 inches color LCD |  |  |  |  |
| Resolution | 240(horizontal) × 320(vertical) pixels |  |  |  |  |
| Color | 65536 color,TFT |  |  |  |  |
| Environment |  |  |  |  |  |
| Temperature | Working Temperature:0℃～40℃ Storage Temperature:-20℃～60℃ |  |  |  |  |
| Relative Humidity | ≤90%RH; no condensation |  |  |  |  |
| Height | 2,000 meters |  |  |  |  |
| Cooling | Natural cooling |  |  |  |  |
| Other |  |  |  |  |  |
| Communication Port | USB communication, compatible with SCPI communication protocol |  |  |  |  |
| Dimension | 82mm(Width)× 142mm(Height)×226mm(Depth) |  |  |  |  |
| Weight | Approx. 1.9 kg |  |  |  |  |

V1.1.2